5V ESP8266 four-way WiFi relay IoT smart home mobile phone APP remote control switch

- Overview

The 5V ESP8266 four-way WIFI relay module uses ESP-01 as the WIFI module and a mature and stable 8-bit MCU chip. It only requires a simple configuration process to achieve wireless control of 4-way relays in the local area network using a mobile phone APP.

- Features

- 1. Onboard high-performance microprocessor and ESP-01 WIFI module;

- 2. The module has 2 working modes :

- Mode 1: The mobile phone is directly mounted on the WIFI module;

- Mode 2: The mobile phone and WIFI module are mounted on the router at the same time;

- Additional function: It can also be used as a USB relay when ESP-01 is unplugged.

- 3. Transmission distance :

- (1) In an open environment, the maximum stable transmission distance when the mobile phone is mounted on the WIFI module is 100m;

- (2) When the WIFI module and the mobile phone are installed on the router at the same time, the transmission distance depends on the signal strength of the router.

- Depends on weakness.

- 4. Use Smartconfig technology to complete the configuration of the ESP-01 WIFI module account and password on the mobile APP. The configured account and password have a power-off memory function;

- 5. Onboard 5V, 10A/250V AC 10A/30V DC relay, which can be connected continuously for 100,000 times, has diode overflow protection and short response time;

6. Onboard mode selection and real-time working status indicator light;

7 , with 4 optocoupler isolation, strong anti-interference ability;

8. Reserve UART debugging interface and MCU program download interface.

3. Hardware introduction and description

Board size: 60 * 63 mm

Board function description:

1. Introduction to onboard resources:

IN+, IN- : 5V power input ;

5V, GND, TX, RX : UART serial port pins;

SWIM, PIN8, NRST: reserved MCU program download port.

Button S1: Mode switching, the default is mode 1

Button S2: Restore factory settings

LED D 1/ D 2/D3/D4 (red light): relay working indicator light, lights up when turned on

LED D7 (red light): Mode 1 indicator

LED D5 (blue light): Mode 2 indicator light

LED D6 (green light): Working status indicator light, described as follows:

(1) When it turns off, it means that it is self-configuring or has lost connection with the router;

(2) When it flashes quickly for 0.5S, it means waiting for the mobile APP to configure the WIFI account and password for the ESP-01 module ;

(3) When it flashes slowly for 2S, it means that the configuration is completed and it is waiting to establish a TCP connection with the mobile phone;

(4) When it is always on, it means the TCP connection has been successfully established with the mobile phone.

The two reserved jumper caps: please insert them both to the bottom during normal use (i.e. RX is connected to RX1, TX is connected to TX1). If you want to use the USB to TTL serial port module alone to debug the ESP-01 module, please plug them both to the top (that is, RX is connected to RX1, TX is connected to TX1). Otherwise there may be interference).

COM 1 : common terminal;

NC 1 : Normally closed terminal, short-circuited with COM 1 before the relay is closed , and left floating after closing;

NO 1 : Normally open, the relay is suspended before closing, and short-circuited with COM 1 after closing .

COM2 : public terminal;

NC2 : Normally closed end, short-circuited with COM2 before the relay is closed , and left floating after closing;

NO2 : Normally open, the relay is suspended before closing, and short-circuited with COM2 after closing .

COM3 : Common port;

NC3 : Normally closed end, short-circuited with COM3 before the relay is closed , and left floating after closing;

NO3 : Normally open, the relay is suspended before closing, and short-circuited with COM3 after closing .

COM4 : Common port;

NC4 : Normally closed end, short-circuited with COM4 before the relay is closed , and left floating after the relay is closed;

NO4 : Normally open, the relay is suspended before closing, and short-circuited with COM4 after closing .

Relay control instructions (hexadecimal hex format ) :

Open the first relay : A0 01 01 A2

Close the first relay : A0 01 00 A1

Open the second relay : A0 0 2 01 A 3

Close the second relay : A0 0 2 00 A 2

Open the third relay : A0 0 3 01 A 4

Close the third relay : A0 0 3 00 A 3

Open the fourth relay : A0 0 4 01 A 5

Close the fourth relay : A0 0 4 00 A 4

Warm reminder: The following is an example of using the mobile phone APP to control the first two relays. The third and fourth relays are used in the same way, but the relay control instructions are different.

2. Prepare the following tools and software before use:

(1) 5V/1A power adapter, the positive and negative poles of the power supply are connected to the IN+ and IN- of the module respectively;

- (2) Install the APP "EspTouch_Demo" on the Android phone, which is used to configure the WIFI account and password for the ESP-01 module when using working mode 2 for the first time;

- (3) Install the APP "EasyTCP_20" on the Android mobile phone , a TCP transmission tool, used to send relay control instructions. Click "SWITCH", then long press the gray square in the interface to enter the name and content of the 4-way relay control instructions (instruction format (HEX form) , taking the previous 2-way relay as an example:

3. Instructions for use in working mode 1 (the mobile phone is mounted on the ESP-01 module) , taking the first 2 relays as an example :

(1) Plug in the ESP-01 module and power on the module. After about 4 seconds, the green light will change from off to slow flashing for 2 seconds, indicating that the configuration is complete, as follows:

- (2) Connect the mobile phone to the AP hotspot signal sent by the ESP-01 module

- (3) Open the "EasyTCP_20" APP, click "CONNECT", enter the IP address: 192.168.4.1 and port number 8080, and then click "Connect". After the connection is successful, the green light will flash slowly from 2 seconds to solid, click the gray square You can send instructions to control the switch of the relay .

4. Instructions for use in working mode 2 (mobile phone and ESP-01 module are mounted on the router at the same time) , taking the first 2 relays as an example :

- (1) Plug in the ESP-01 module, power on the module, wait for the green light to flash slowly, and then press the S1 key to switch to mode 2. The blue light will light up. After about 1 minute, the green light will change from extinguishing to 0.5S fast flashing. Description: Waiting for "EspTouch_Demo" APP to configure WIFI account and password for it

- (2) At this time, the mobile phone is connected to the router first, open the "EspTouch_Demo" APP, enter the router password, and click "Confirm"

- Wait for the configuration to be successful. When the IP address of ESP-01 (such as 192.168.0.189) appears on the APP interface, it means that the ESP-01 module has successfully connected to the router and automatically memorized the account and password. It will automatically connect next time it enters mode 2 ( It will take about 20-60S to connect)

- It should be noted that the IP address 192.168.0.189 is dynamically assigned to the ESP-01 module by the router. The address may change after the next reconnection. You can check the real-time IP of the ESP-01 module in the device list of the router. address.

- (4) Open the "EasyTCP_20" APP, click "CONNECT", enter the IP address of the ESP-01 module: 192.168.0.189 and port number 8080, then click "Connect". After the connection is successful, the green light will flash slowly for 2 seconds. Always on, click the gray square to send instructions to control the relay switch.

5. Additional function (as USB relay) instructions for use:

- Prepare a USB to TTL serial port module. Connect the GND, TX, and RX of the TTL module to the GND, TX, and RX of the relay module respectively. Unplug the ESP-01 module, select mode 1, and wait for the green light to change to 2S slow flashing on the computer. Open the serial port debugging software (such as SSCOM32 ) , select the baud rate of 1152 00, and send the relay control command in hexadecimal (hex) form to open or close the corresponding relay. Taking the first relay as an example, send A0 01 01 A2 can open the first relay :

Kind tips:

1. It will take a little longer to configure the WIFI password for the first time (about 1 minute). After the configuration is completed, it will only take 20 seconds to connect automatically the next time you turn on the phone.

2. If you want to replace the router, you can power off and restart or press the S2 key (pressing the S2 key will clear the previously memorized WIFI account and password) , and re-configure the WIFI account and password for ESP-01 in mode 2 .

3. When the signal of the router memorized by ESP-01 is very weak or is not in the service area, causing the connection to be interrupted, the green light will go out and the connection will be automatically attempted. During this process, the buttons will be in an invalid state. When the green light flashes slowly for 2 seconds, the connection has been restored. .

4. In Mode 1 and Mode 2, the buttons are only available when the green light flashes slowly for 2 seconds or is always on. In other cases, the buttons are invalid due to internal self-configuration of the chip or waiting for the configuration process.

5. ESP-01 has a timeout mechanism. If there is no data exchange between the mobile phone and ESP-01 for more than 6 minutes, the TCP connection will be automatically disconnected. At this time, click " CONNNECT" in the upper right corner of the "EasyTCP_20" APP to re-establish the connection.

6. Regarding the onboard UART interface: when debugging 8266, the TX, RX, and GND of the USB to TTL module are connected to the RX, TX, and GND of the relay module respectively; when the relay module is used as a USB relay, the USB to TTL module TX, RX, and GND are connected to the TX, RX, and GND of the module respectively.

7. Regarding the reserved MCU program download port: when the onboard MCU is STM8S003/STM8S103, NRST and SWIM are the programming interfaces; when the onboard MCU is N76E003, NRST, SWIM, and PIN8 respectively correspond to the Nu-Link programmer. RST, CLK, DAT interface.

8. The serial port baud rate of the onboard MCU is 115200, so when the APP cannot connect to the ESP-01, please first ensure that the baud rate of the ESP-01 module used is also 115200.